EARLY LOCOMOTIVES USED IN THE BLUE MOUNTAINS

Rev  5/14

Passenger Locomotives

1865.  G(23)  Class  2-4-0  Beyer Peacock Manchester.                                              13 in Class

The G23 class was the first passenger locomotive to use the Lapstone Zig Zag in June 1867 after the completion of the Victoria Bridge over the Nepean River at Penrith. The first nine locomotives in the class had a driving wheel diameter of 69.5 inches which proved to be unsuccessful for  Blue Mountains work so the final four in the class were manufactured with wheel diameter of 66 inches. These final models had a tractive effort of 11,310 lbs against 10,740 for the early models. The latter four units Numbers 32 to 35 which served from  1870to 1874 working the Penrith to Mount Victoria "Daily Mixed".

Never popular they suffered from hot axleboxes on their heavily loaded two wheel Bissel  Truck. They were replaced by the 79 class.

1877 C(77) Class  4-4-0                                                                                                         68 in Class

A further development of the Beyer Peacock 23 class. They had a four wheel leading Bissel bogie and 20 percent more power with a tractive effort of 13,000 lbs. They took over passenger and mail services for 20, years. They were the first locomotives to be fitted with Westinghouse air brakes.

The run to Mount Victoria took 4 hours and 30 minutes climbing the Zig Zag and stopping at all stations beyond Parramatta.